|  | # Mechanicsville Volunteer Fire Department, Inc. # Standard Operating Guidelines |  |
| --- | --- | --- |
|  | # Subject: Engine/Rescue Company Operations # & Riding Assignments | # SOG # 2-07 |
|  |  | Initiated 01/01/97 |
|  | Approved: Fire Chief John B. Raley, Jr. | #### Revised 08/23/03 |

***CURRENTLY UNDER REVISION***

- PURPOSE

1.	The purpose of this procedure is to establish assignments for the engine & rescue company on the fire ground.

  - This procedure is to provide the crew of the engine & rescue company, the basic tasking and equipment needed to carry out assignments.

- RESPONSIBILITIES

1.	All firefighters confer with the engine or rescue officer for any additional information and/or specific assignments. The portable radio shall go to a senior firefighter. The portable radio may go to the layout man if the officer goes in with the crew. Tasks and tools may be altered due to manning limitations. A request to command for additional manpower might be warranted.

2.	The driver/operator can be utilized on exterior assignments provided he/she has the proper turnout gear. (helmet, coat, gloves and adequate foot wear) Examples are; ladders and the placement of same, horizontal ventilation of windows, tool staging and scene lighting)

3.	The officer will oversee the engine or rescue crew progress, assist as required and report the same to command. The officer should be the officer in charge of firefighting/rescue operations.

4.	Firefighters shall report to the engine or rescue officer the completion and/or progress of assigned task. Firefighter assignments are listed below.

| ENGINE-21 | # SQUAD-2 | ENGINE-23/24/222 |
| --- | --- | --- |
| DRIVER OFFICER 1 2 | DRIVER OFFICER 1 2 | DRIVER OFFICER 1 2 |
| 3 4 5 6 | 3 4 5 6 7 | 3 4 5 6 |

7.	The following are general rules for the engine company:

a.	The layout person will be preferably a probationary member or the newest member riding the apparatus.

- Firefighters will exit engine with caution. - WATCH FOR TRAFFIC –

- Firefighter connects the humat valve to hydrant unless otherwise instructed by the officer. He/she should have portable radio if possible. (NOTE: Hydrant areas)

d.	Firefighter DOES NOT charge supply line if NOTHING SHOWING.

e.	Firefighter DOES charge supply line if SMOKE/FIRE SHOWING or the engine company goes out of sight and the firefighter is unsure and/or does not have a portable radio.

f.	After connecting the humat valve to the hydrant and following above guideline, the firefighter will report to the scene for further orders from the engine company officer.

g.	The remaining firefighters will stay on the apparatus and wait for instructions from the officer.

h.	All firefighters not wearing SCBA equipped with automatic pass devices will turn 	their “PASS” alerting devices on before they breath air.

i.	When instructions are received from the officer, the crew should exit the crew cab from the right side if possible.

j.	For first due structure fires, all firefighters will mask up.

k.	For all automobile fires, at least two firefighters will mask up.

- For mutual aid structure fires, all firefighters will mask up.

ENGINE 21 RIDING ASSIGMENTS

| ENGINE-21 |  |  |  |
| --- | --- | --- | --- |
| Driver 1 |  | Officer 2 |  |
| 3 | 4 | 5 | 6 |

## POSITION 1 – DRIVER

- Operate apparatus in a safe manner.

- Pump operations

- Provide scene lighting

#### POSITION 2 - OFFICER

###### Placement of the apparatus

- Scene size-up

- Attack line placement

##### POSITION 3 – LAYOUT

- Layout the 4” supply line

- Assist with second attack/back-up line

POSITION 4 – 2ND LINEMAN

- Pull back-up line

##### POSITION 5 – BACK-UP LINEMAN

- Assist with initial attack line

- Flake out hose and back-up lineman

##### POSITION 6 – LINEMAN

- Pull initial attack line

### ENGINE 21 RIDING ASSIGNMENTS FOR A 4 PERSON CREW

| ENGINE-21 |  |  |  |
| --- | --- | --- | --- |
| Driver 1 |  | Officer 2 |  |
| 3 |  |  | 4 |

## POSITION 1 – DRIVER

- Operate apparatus in a safe manner

- Pump operations

- Provide scene lighting

##### POSITION 2 – OFFICER

- Placement of apparatus

- Scene size-up

- Attack line placement

##### POSITION 3 – LAYOUT

- Layout the 4” supply line

- Assist with the attack line

##### POSITION 4 – LINEMAN

- Pull attack line

SQUAD 2 RIDING ASSIGMENTS

| Driver 1 | # SQUAD-2 | Officer 2 |
| --- | --- | --- |
| 3 |  | 4 |
| 5 | 6 | 7 |

POSITION 1 – DRIVER

## Operate apparatus in a safe manner

- Pump operations

- Generator operation, turn on power units

- Provide scene lighting

### POSITION 2 – OFFICER

#### Placement/positioning of apparatus

- Scene safety and size-up

- Tool placement

### POSITION 3 – TOOLMAN

#### Pull Extrication Tools

### POSITION 4 – VEHICLE SAFETY

#### Grab extrication jump bag

- Secure power/hazards on/under vehicle (absorbent)

- Assist with extrication tools

### POSITION 5 – LINEMAN

#### Pull and man trash line with SCBA

### POSITION 6 – MEDICAL

#### Grab medical & “C” collar bag

### POSITION 7 – STABILIZATION

- Grab step chocks & stabilize vehicle(s)

- Assist with extrication tools

- Assist with medical if necessary

SQUAD 2 RIDING ASSIGMENTS (STRUCTURE FIRES)

| Driver 1 | # SQUAD-2 | Officer 2 |
| --- | --- | --- |
| 3 |  | 4 |
| 5 | 6 | 7 |

POSITION 1 – DRIVER

###### Operate apparatus in a safe manner

- Generator operation

- Provide scene lighting

- Cascade system operation

POSITION 2 – OFFICER

- Scene size-up

- Search & Rescue

- Handlight

POSITION 3 – SEARCH & RESCUE

###### Grab Hook and water can

- Search & Rescue

- Salvage & Overhaul

POSITION 4 – FORCIBLE ENTRY

- Axe & Halligan (married)

- Handlight

- Search & Rescue

- Salvage & Overhaul

POSITION 5 – SEARCH & RESCUE

###### Handlight

- Grab Hook

- Search & Rescue

- Salvage & Overhaul

POSITION 6 – VENTILATION

- Ventilate (horizontal and or vertical)

- Secure power to the establishment at the panel

- Fans

- Search & Rescue

- Salvage & Overhaul

POSITION 7 - VENTILATION

- Ventilate (horizontal and or vertical)

- Secure power to the establishment at the panel

- Fans

- Search & Rescue

- Salvage & Overhaul

NOTE: PRIMARY DUTIES – SEARCH & RESCUE – SALVAGE & OVERHAUL

ENGINE 23, ENGINE 24 & ENGINE 222 RIDING ASSIGMENTS

| 1 |  | 2 |  |
| --- | --- | --- | --- |
| 3 | ## ENGINE-23 ## ENGINE-24 & ## ENGINE-222 |  | 4 |
| 5 |  |  | 6 |

# POSITION 1 – DRIVER

- Operate apparatus in a safe manner

- Pump operations

- Provide scene lighting

#### POSITION 2 – OFFICER

- Placement of the apparatus

- Scene size-up

- Attack line placement

POSITION 3 –BACK-UP LINEMAN

- Assist with initial attack line

- Flake out hose and back-up lineman

##### POSITION 4 – LINEMAN

- Pull initial attack line

##### POSITION 5 – LAYOUT

- Layout the 4” supply line

- Assist with second attack/back-up line

#### POSITION 6 – 2nd LINEMAN

- Pull a back-up line

ENGINE 23, ENGINE 24 & ENGINE 222 RIDING ASSIGMENTS

FOR A 4 PERSON CREW

| 1 |  | 2 |  |
| --- | --- | --- | --- |
| 3 | ## ENGINE-23 ENGINE-24 & ## ENGINE-222 |  | 4 |

###### POSITION 1 – DRIVER

- Operate apparatus in a safe manner

- Pump operations

- Provide scene lighting

###### POSITION 2 - OFFICER

- Placement of apparatus

- Scene size-up

- Attack line placement

POSITION 3 – LAYOUT

- Layout the 4” supply line

- Assist with the attack line

POSITION 4 – LINEMAN

A) Pull initial attack line